REPORT: Analysis of the influence of different artifacts in the video

content from three experiments.

Alexandre Fieno da Silva1, Judith A. Redi2, and Myle`ne C.Q. Farias1

1University of Brasilia, Campus Universitario Darcy Ribeiro, Brasilia - DF, Brazil, 70919-970

2Delft University of Technology, Mekelweg 4, Delft, The Netherlands, 2628 CD

In this project, we are interested in understanding the impact of a set of artifacts on the quality of video, their relationship with the content and to determine their impact on visual quality  perception. The set of artifacts  used in this study was chosen among those most perceptually  relevant for digital  video applications (e.g. blockiness, blurriness,  packet-loss etc). We first plan to perform a series of psychophysical experiments  on visual quality  perception. These experiments will determine the impact on quality first of a set of spatial and temporal artifacts by themselves, and later of their combinations, taking into account different  content with varying spatial and temporal characteristics.

Index Terms—Quality of video, visual quality  perception, blockiness, blurriness and packet  loss.

I.  INTRODUCTION

NE approach to design video quality metrics is to use artifact metrics. They have several  advantages, for ex-

O

ample simplicity and the ability to provide a good description of what is wrong (or right) with the video. But, in order to design  good artifact metrics or any quality metric, it is necessary to have a good understanding  of how artifacts affect quality. Therefore, it is important to study the characteristics of the most relevant  artifacts  present in digital video, what includes characteristics of annoyance, visibility, interactions among artifacts and content dependence. This way, we can use this data together with the data gathered previous to determine annoyance, visibility, and interaction for a diverse  content.

In this experiment, we focus on 3 particular artifacts that can be considered among the most relevant for digital video: blurriness, blockiness,  and packet loss. We  present  these artifacts at several strengths, in several combinations  and for seven original videos. In summary, we are interested in un- derstanding ”how” artifacts combine to determine annoyance and degradation in order to design better metrics.

A. Previous Experiments

1) Experiment 1

In Experiment 1 (Exp. 1), all videos were encoded using the H.264AV C codec at a very high bitrate (approximately

120 Mbps), with 8 packets  per frame. The coding process generated three sequences per video,  with a Group of Pictures (GOP) size set  to 4, 8 and 12, respectively.  Packet  loss artifacts were then generated by dropping  data packets from the bitstream [2]. The packet loss ratios for all videos were

0.7%, 2.6%, 4.3% and 8.1%. In total, it  was  generated  7

videos x 3 GOP x 4 packet loss ratios = 84 sequences. The

7 pristine videos were added to the set, making  a total of 13 different settings per original video and resulting in 91 test sequences [2].

2) Experiment 2

Experiment 2  (Exp. 2)  is  composed  of  two  artifacts (blockiness and  blurriness) considered the  most  salient present  in digital videos [8].  This experiment  follows the ITU-T Recommendation  P.930 that suggests to introduce blockiness  only in regions where these  artifacts are more visible. Thus, blockiness  artifacts were produced  by using the difference  between the average of each 8 × 8 block and the average of the 24 × 24 surrounding  area (which has the current 8 × 8 block as a center)  to make each block stand out. Recommendation  P.930 suggests the generation of blurriness with the use of a low-pass  filter [8]. Thus, blurriness artifacts were generated  by applying a symmetric,   two dimensional finite duration  impulse  response (FIR) low-pass filter to the digital  image array. Different filters with  varying cut-off frequencies could be used to control the amount of blurriness introduced.  The blockiness  and blurriness  strengths  for all videos were 0.4, 0.6 and 0.8. In  total, it  was  generated

7 videos × 10 blockiness  and blurriness strengths  =  70 sequences.  The 7 pristine videos were added to the set, resulting in 77 test sequences [4].

3) Experiment 3

In Experiment 3 (Exp. 3) has been considered  all original videos of the previous experiments. Also, it is composed of the three artifacts  has been used in previous experiments (i.e., blockiness,  blurriness  and packet loss). In Exp. 3 we are interested to understanding the impact on perceived  quality of combinations of these artifacts  and their relationship with the content. In total, we used 140 test sequences and, details are given in the sub-section II-B.

II.  EXPERIMENTAL SETUP

A. Video Database

The video database used in this experiment  was generated from seven high-definition  videos (the same videos have been used in previous experiments). All them with 1280 × 720, 50

fps and 10 seconds duration.  Representative frames of these originals  are shown in Figure 1.

Fig. 1.  First frame of the sequences included  in Experiment.

B. Artifacts

New versions of each original video were generated using different combinations of artifacts. To generate test sequences, we generate blockiness, blurriness and packet loss artifacts and combine  these artifacts at different  proportions and strengths. In summary, the following parameters are used to generate the test sequences:

• 7 original videos Basketball, Romeo and Juliet, Park Run, Cactus, Park Joy, Barbecue and Into Trees;

• 4 different strengths of blockiness and blurriness: 0.4, 0.6

and 0.8;

• 3 different packet loss ratios: 0.7%, 2.6%, 4.3% and

8.1%.

Due to a  large number of  combined test videos (over

200 videos) makes  it impossible to perform an experiment in a  reasonable  amount  of time. Therefore,  a reduction  of the number  of test sequences was  achieved  by selecting   a subset of the original group. In this process, we eliminated most of  the sequences  already used in  the two previous experiment already performed. We also reduced the number of parameters considering the results of the previous experiments. In summary, to select the test sequences we considered the following:

1) Total time necessary to execute the overall experiment;

2) The results  of two previous  experiments  showed that the mean annoyance and the mean strength  scores are significantly different for packet loss ratios of 0.7 and

8.1.

3) Among the blockiness  and blurriness  strength values considered in the previous experiments, we chose only the strength 0.4 and 0.6, which were considered to be more representative of these artifacts.

Thus, taking into account these choices, they resulted into

19 combinations (Table I) × 7 (video versions)  + 7 orig- inal videos = 140 videos. This is a reasonable  number  of sequences which made it possible to run the experiment in the allocated time of 50 minutes.

C. Methodology

For all experimental  session, subjects are requested to score a set  of test videos with different combinations of artifacts. The number of observers was chosen in accordance with ITU

TABLE I

COMBINATIONS OF THE PACKET LOSS RATIO (0.7 AND 8.1) WITH

BLOCKINESS AND  BLURRINESS (STRENGTHS 0.4 AND 0.6) ARTIFACTS.

| Combination | Packet-Loss | Blocky | Blurry |
| --- | --- | --- | --- |
| 1 | 0.0 | 0.6 | 0.0 |
| 2 | 0.0 | 0.0 | 0.6 |
| 3 | 8.1 | 0.0 | 0.0 |
| 4 | 0.7 | 0.0 | 0.4 |
| 5 | 8.1 | 0.0 | 0.4 |
| 6 | 0.7 | 0.0 | 0.6 |
| 7 | 8.1 | 0.0 | 0.6 |
| 8 | 0.7 | 0.4 | 0.0 |
| 9 | 8.1 | 0.4 | 0.0 |
| 10 | 0.7 | 0.4 | 0.4 |
| 11 | 8.1 | 0.4 | 0.4 |
| 12 | 0.7 | 0.4 | 0.6 |
| 13 | 8.1 | 0.4 | 0.6 |
| 14 | 0.7 | 0.6 | 0.0 |
| 15 | 8.1 | 0.6 | 0.0 |
| 16 | 0.7 | 0.6 | 0.4 |
| 17 | 8.1 | 0.6 | 0.4 |
| 18 | 0.7 | 0.6 | 0.6 |
| 19 | 8.1 | 0.6 | 0.6 |

Recommendations [9]. A Single Stimulus setup with implicit reference  [9]  was  adopted  for the task. The experiment  is running in a room with constant illumination of approximately

70 lux. Each subject watches the stimuli (test sequences) on a

23 inches LED monitor of resolution 1360 × 768. The distance between the subject’s  eyes and the video monitor is 3 times the monitor screen’s height.

Subjects are seated straight  ahead of the monitor, centered at or slightly below the eye height for most subjects. A chinrest is used to guarantee a constant distance between the subject’s eyes and the monitor. A SensoMotoric Instruments iView RED Eye Tracker  was used throughout  the experiment to record the eye-movements of the participants  (The eye tracker had a sampling rate of 50/60Hz, a pupil tracking resolution of 0.1◦ and a gaze position  accuracy of 0.5 − 1◦ ). The user interface for the experiment is implemented using the Neurobehavioral Systems software  Presentation.

All  subjects  are asked  to watch the test sequences  and indicate whether they perceived any impairment in the videos; if  so, they were asked  to enter how annoying the artifacts were on a  continuous   annoyance  scale ranging between  0 to 100. During the experiment, the experimenter  guides the subject through all sessions (calibration, free viewing, training, practice and a main experiment).  Table II show the summary of the settings of each experiment.

III.  DATA ANALYSIS

We  used the standard methods  [9] for analyzing the an- noyance  judgments  provided by the test subjects. We  first computed Mean Opinion Score (MOS) for each test sequence. MOS is calculated by averaging  the annoyance  levels over all observers for each video. In this report MOS’s values are treated as MOS1, MOS2 and MOS3 (i.e., MOS of the Exp.1, Exp.2 and Exp.3, respectively). To test the relationship  among the MOS’s  values  we used  the Spearman’s correlation  (ρ). To determine the significant effects between  MOS’s  values

TABLE II

SUMMARY OVERALL  EXPERIMENTS

| Settings | Exp. 1 | Exp. 2 | Exp. 3 |
| --- | --- | --- | --- |
| Numbers of Test Sequences | 91 | 77 | 140 |
| Numbers of Subjects | 15 | 16 | 23 |
| Calibration | − | Yes | Yes |
| Free Viewing | Yes | Yes | Yes |
| Training | Yes | Yes | Yes |
| Practice Trials | Yes | Yes | Yes |
| Main Experiment | Yes | Yes | Yes |
| Packet Loss | Yes | − | Yes |
| Blockiness | − | Yes | Yes |
| Blurriness | − | Yes | Yes |

we used a non-parametric  T-Test.  We also are analysing  only where settings (artifacts)  are the same among the experiments:

• Case  1: from Exp. 1 until 3 considering only original videos;

• Case 2: between Exp. 1 and Exp. 3, considering only the

8.1% ratio of packet loss artefact;

• Case  3: between Exp. 2 and Exp. 3, considering only the 0.6 strengths  of blockiness and blurriness artifacts (isolated).

A. Case 1

We report the MOS’s  computed  across all 7 versions  of video and all experiments,  as shown in Figure 2. It is interest- ing to notice how the MOS’s changes among the experiments: the MOS’s of the experiments 1 until 3 are 1.43, 4.92 and 0.80, respectively. To know the strength and direction of association that exists  among  those MOS’s,  the Spearmans  correlation has been calculated. Results has showed that exist a moderate correlation (ρ = .595) between M OS1 and M OS3. In other cases, there  was one very weak correlation  between M OS1 and M OS2 (ρ = −.214) and, between M OS2 and M OS3 (ρ = .090). Although, there are not statistically significant different (p > .05) among them.

B. Case 2

In this case, we report the M OS1 and M OS3 computed to packet loss artefact across all 7 versions of video, as shown in Figure 3. It is possible to notice that M OS1 = 72.85 is very higher than M OS3 = 37.99 with ρ = .536 and p < .05.

C. Case 3

Two different analyses  were made between  M OS2 and M OS3 across  all 7 versions  of video: first, we report the MOS’s  computed  only to blockiness  artefact  and next, we report the MOS’s  computed only to blurriness artefact. Fig- ure 4 shows a slightly difference between M OS2 and M OS3 to Park Joy, Into Tree and Park Run videos, but this differ- ence increased  when to Romeo & Juliet, Cactus, Basketball and Barbecue videos. Spearman’s correlation  coefficient  has showed that there is  a  moderate   and positive  correlation (ρ = .536) between  them. Although, there are statistically significant different only to Romeo &  Juliet, Cactus and Basketball videos (p < .05).

Fig. 2.  Mean Annoyance  Values averaged across all the original versions of each video.

Fig. 3.  Mean Annoyance  Values averaged across of the packet loss versions of each video.

Next, it is considered blurriness artefact. As shown in Fig- ure 5 there is a slightly  difference between M OS2 and M OS3 to Park Joy, Into Tree, Cactus, Basketball and Barbecue videos increasing  this different to Park  Run and Romeo  &  Juliet videos. Despite of strong correlation (ρ = .857) between them, there was not statistically significant different (p > .05).

IV.  CONCLUSION

The conclusion  goes here.

Fig. 4.  Mean Annoyance  Values averaged across of the blockiness versions of each video.

[4]  Farias, M. C. Q.; Foley, J. M.; Mitra, S. K., Some Properties  of Synthetic Blocky and Blurry Artifacts. In: Proceedings of the SPIE, Human Vision and Electronic Imaging VIII, Vol. 5007, pp. 128-136 (2003).

[5]  Farias, M. C. Q.; Foley,  J. M.; Mitra, S. K., Perceptual analysis of video impairments that combine  blocky, blurry, noisy, and ringing synthetic artifacts. In: Proceedings  of the SPIE, Human Vision and Electronic Imaging VIII, Vol. 5666, pp. 107-118 (2005).

[6]  Farias, M. C. Q.; Foley, J. M.; Mitra, S. K., Detectability  and Annoyance of Synthetic Blurring and Ringing in Video  Sequences. In: International Conference on Acoustics, Speech, and Signal Processing - ICASSP, Vol.

3, pp. 697-700, 2004.

[7]  Farias, M. C. Q.; Foley, J. M.; Mitra, S. K., Detectability  and Annoyance of Synthetic Blocky and Blurry Video Artifacts. In: Journal Sid Symposium Digest of Technical  Papers, Vol. 33, No. 1, 2002.

[8]  Farias, M. C. Q.; Foley, J. M.; Mitra, S. K., Detectability  and annoyance of synthetic blockiness, blurriness,  noisiness, and ringing in video se- quences. In: Proc. IEEE International Conference on Acoustics,  Speech, and Signal Processing, Philadelphia, PA, Vol. 2, pp. 553556, 2005.

[9]  International Telecommunication Union, ITU-T Recommendation BT.500-

8: Methodology for the subjective  assessment of the quality of television pictures. 1998.

[10]  Farias, M. C. Q.; Mitra, S. K., No-reference video quality metric based on artifact measurements. In: ICIP, IEEE, pp. 141-145, 2005.

[11]  Landau, S.; Everitt, B. S., A Handbook of Statistical  Analyses Using

SPSS. ISBN: 9781584883692,  Statistics Chapman & Hall/CRC, Taylor

& Francis, 2004.

[12]  Montgomery, D., C., Introduction To Statistical Quality Control. ISBN:

9780470233979,  John Wiley & Sons, Incorporated,  2009.

[13]  Seshadrinathan, K.; Soundararajan, R.; Bovik, A. C.; Cormack, L. K., Study of Subjective and Objective Quality Assessment of Video. In: IEEE Transactions on Image Processing, Vol. 19, No. 6, Piscataway, NJ, USA,

2010.

Fig. 5.  Mean Annoyance  Values averaged across of the blurriness versions of each video.

REFERENCES

[1]  Farias, M.  C. Q.; Heynderickx, I.;  Bruno, L.  M.  E.; Redi, J.  A., Visual Artifacts Interference Understanding and Modeling (VARIUM). In: Seventh International Workshop on Video Processing and Quality Metrics for Consumer Electronics, 2013, Scottsdale.

[2]  Redi,  J. A.; Heynderickx, I.; Bruno, L. M. E.; Farias, M. C. Q., On the impact of packet-loss impairments  on visual attention  mechanisms. In: ISCAS, pp. 1107-1110. IEEE, 2013.

[3]  Farias, M. C. Q.; Foley,  J. M.; Mitra, S. K., Perceptual contributions of blocky, blurry and noisy artifacts to overall annoyance. In: Proc. IEEE International Conference on Multimedia & Expo, Baltimore, MD, USA,

2003, Vol. 1, pp. 529532.